Project Title- HEALTH AND MOVEMENT MONITORING SYSTEM FOR SOLDIERS

Paper Code-KEC-753

Semester: VII

in

I.T.S Engineering College Gr. Noida

Submitted for partial fulfilment of award of

BACHELOR OF TECHNOLOGY

Degree in

Electronics & Communications Engineering

Synopsis Submitted by

| S. NO | STUDENT NAME | STUDENT ROLL NO. | MOBILE NO. |
| --- | --- | --- | --- |
| 01. | AMBIKA | 2002220310004 | 7289892862 |
| 02. | RAHUL RAJ | 2002220310016 | 7283083763 |
| 03. | RAJU KUMAR | 2002220310017 | 7562838529 |

Under the Guidance of 
(Asst. Prof. Mr. Agha A. Hussain )

# DEPARTMENT OF ELECTRONICS & COMMUNICATION  ENGINEERING

# I.T.S ENGINEERING COLLEGE, Gr. Noida

# 46, KNOWLEDGE PARK-III, GREATER NOIDA

Affiliated from Dr.  A.P.J Abdul Kalam Technical University, Lucknow Session 2020-2024

# INTRODUCTIONS

# The Soldier Health Monitoring and Position Tracking System allows the military personnel to track the current GPS position of a soldier and also checks the health status including body temperature and heartbeat of a soldier.

# To address this critical issue, the Indian Army has been implementing advanced technologies such as satellite communication systems, drones, and wearable devices to ensure real-time monitoring and transmission of vital health data from the soldiers in the field to the army base stations. These technologies not only enhance situational awareness but also enable prompt medical intervention and evacuation in case of emergencies, thereby significantly improving the safety and well-being of our brave soldiers. The timely adoption of these technologies is a crucial step towards strengthening the nation’s defense capabilities and honoring the sacrifices made by our soldiers.

# During any special operation or mission that’s been carried out by these services, soldiers involved tends to get injured or get lost on the battlefield. As the soldier plays a significant role in national security, we cannot afford to let them get lost, or have any delayed medics reach the injured ones. So, to protect these soldiers we should have some technology that monitors and tracks the soldiers in real-time and help minimize the time of search operation, and rescue operation efforts of the control unit.

# So, to support this idea, the project presents an effectual system that is capable of monitoring the health vitals of soldiers and at the same time able to record their current position using necessary sensors. The data collected from the sensors are then transmitted to the next level of the hierarchy using wireless RF modules.

# This system enables the control room unit to track the location and monitor the health vitals of the soldiers constantly using the wireless body sensor network, and the GPS receiver at frequent intervals. Arduino associated with the control room constantly monitors the data received from the different subsystems of the proposed design and triggers a warning if any values cross the set threshold values. Live health monitoring and position tracking of the soldiers will ensure that they are safe on the battlefield and if any abnormalities are found in the values received to the control unit node, they make sure that relief is sent from the control unit node or the squadron leader’s node within a minimum amount of time.

# LITERATURE SURVEY

# PURPOSED METHODOLOGY USED

# Components Required

# Arduino UNO (x3)

# Temperature Sensor - LM35 (x2)

# Pulse Sensor - RC-A-4015 (x2)

# GPS Receiver - Neo-6M (x2)

# HC-12 RF module

# SX-1278 RF LoRa module

# 9V 3A Battery

# Breadboard and jumper wires

# Components Descriptions

# HC-12 Sensor

# HC-12 wireless serial port communication module is a new-generation multichannel embedded wireless data transmission module. The HC-12 is a half-duplex 20 dBm (100 mW) transmitter paired with a receiver that has -117 dBm (2×10-15 W) sensitivity at 5000 bps. Paired with an external antenna, these transceivers are capable of communicating up to and possibly slightly beyond 1 km in the open and are more than adequate for providing coverage throughout a typical house. Its wireless working frequency band is 433.4-473.0MHz, multiple channels can be set, with the stepping of 400 KHz, and there are totally 100 channels.

# SX1278

# The SX1278 transceiver feature the LoRa long range modem that provides ultra-long range spread spectrum communication and high interference immunity whilst minimizing current consumption. Using Semtech’s patented LoRa modulation technique SX1278 can achieve a sensitivity of over -148dBm using a low cost crystal and bill of materials. The high sensitivity combined with the integrated +20dBm power amplifier yields industry leading link budget making it optimal for any application requiring range or robustness. LoRa also provides significant advantages in both blocking and selectivity over conventional modulation techniques, solving the traditional design compromise between range, interference immunity and energy consumption.

# WORKING

# The system has two sections, hardware and software. The system consists of 3 nodes i.e Soldier’s node, Squadron leader’s node, and Control unit node. At all the nodes Arduino constantly monitors and records data from the several sensors connected to the system, and communicates with other nodes using wireless RF modules present in the system.

# The hierarchy of obtaining data from the soldier is divided into three segments:

# 1. Soldier's Node - Level 1

# 2. Squadron's Node - Level 2

# 3. Control unit Node - Level 3

# The block diagram below explains how the nodes and all the internal sensors are connected.

# Circuit Diagram of Project:

# A breif explaination of all the nodes are given below:

# Soldier's Node

# This node comprises of body sensor networks (BSN) i.e a temperature sensor and a pulse sensor. These sensors are used to monitor the health vitals of soldiers. Temperature sensor (LM35) is used for monitoring the temperature of the soldier and his surrounding environment, and the pulse sensor (RC-A-4015) monitors the pulse rate of the soldiers as beats per minute (BPM). If there is any variance among the sensed values and the defined threshold values, then it is considered as an exigency. Along with the BSN, a GPS receiver is also added to the node, so as to locate/monitor the soldier’s position.

# All the data incoming from the sensors are processed and recorded by the micro controller and sent via the wireless RF transceiver module present in the system to the next node i.e Squadron leader’s node. The RF module used in the soldier’s node is HC12, as the communication range between soldier’s node and the squadron leader’s node is short as they are hardly few kms apart.

# Additionally, to connect this node to the internet, we use the Blynk application. With the help of a USB and the laptop, we are able to connect this node to the internet. While this node is connected to the internet, the data from the sensors are uploaded to the Blynk cloud, and concurrently sent to the dashboard created to interface with the project via the cloud.

# Squadron's Node

# This node also comprises of body sensor networks (BSN) i.e a temperature sensor and a pulse sensor. These sensors are used to monitor the health vitals of the Squadron leader. Temperature sensor (LM35) is used for monitoring the temperature of the squadron leader and his surrounding environment, and the pulse sensor (RC-A-4015) monitors the pulse rate of the squadron leader as BPM. If there is any variance among the sensed values and the defined threshold values, then it is considered as an exigency.

# All the data coming from the sensors are processed and recorded by the micro controller and sent via the wireless RF transceiver module present in the system to the final node i.e Control unit node. Before sending the recorded data, foremost thing done by this node is receiving the data sent from the soldier’s node through the HC12 module present in the system.

# Subsequently after receiving the soldier’s data, both the data combined together i.e soldier’s data and the squadron leader’s data are sent through the RF module present in the system. Now, for sending the data to the control unit node, which is very far away from the squadron leader’s node, we use wireless RF Long Range (LoRa) module i.e SX1278, using this RF module we are easily able to send the data of both the soldier’s and the squadron leader to the control unit node which is up to ~ (25kms - 30kms) apart from each other.

Control Unit Node:

# This node includes the wireless RF LoRa module i.e SX1278, where it receives the data of both nodes i.e soldier’s and squadron leaders, from the squadron leader’s node. This node is just about monitoring, storing, and analyzing the received data from the other two node i.e Soldier’s node and the squadron leader’s node.

# Additionally, the control unit node is provided with an extra feature, where they can monitor soldier’s directly through the internet using a graphical interface/dashboard. This is possible using the blynk application on the mobile phone available at control unit node, provided the soldier’s module is also connected to the internet.

APPLICATIONS

# * Defence Forces

# The project can be implemented in battle field or high altitude areas where health and location of soldiers is the most basic information which should be   known to the control room.

# Civilians

# This project can also be utilized by individuals who work in remote areas or high altitudes wherein the most basic information should be known to someone dear to them or their guardian’s.

# PROJECT TIMELINE

# CONCLUSION AND FUTURE SCOPE

# Soldier’s being an important part of our nation’s security, their health vitals and their location needs to be regularly monitored for their safety and their efficient working. Wireless communication with IoT makes the whole experience of monitoring soldiers health vitals and their location, smart efficient and fast. Technologies like IoT has fundamentally reformed the way we live and work, it has made our life easier.

# Above proposed system not only monitors real-time health and location data of the soldiers but also provides necessary data to help us determine the last position and the health data of the soldiers if he gets lost, so as to track him down and send help.

# This system increases the efficiency of the soldiers working in the field and also reduces the effort of search and rescue operation led by control room unit at the same time. All in all, the system being compact and less in weight makes it more ideal for the soldier’s and the squadron leaders to carry the system with them, which in turns helps the control unit node to monitor their data of in real-time, even if they are apart.

# There is an enormous horizon of possibilities which could be offered - new designs, advanced system could be introduced to improve the conditions and efficiency of the system, and by introducing AI/ML algorithms we can predict out in near future if any soldier might require medical attention beforehand, other than this it can also predict whether a component or sensor might need attention or replacement. Other improvements that can be included is implementing advanced security protocols/techniques on the wireless RF modules so as to make the whole process of transmission and receiving sensors data more secure and reliable.

# REFERENCE

# [1] Hock Beng Lim, Di Ma, Bang Wang, Zbigniew Kalbarczyk, Ravishankar K. Iyer, Kenneth L. Watkin (2010) “A Soldier Health Monitoring System for Military Applications” International Conference on Body Sensor Networks, pp: (246-249).

# [2] William Walker, A. L. Praveen Aroul, Dinesh Bhatia (2009) “Mobile Health Monitoring Systems” 31st Annual International Conference of the IEEE EMBS, Minneapolis, Minnesota, USA, pp: (5199-5202).

# [3] M. Pranav Sailesh, C. Vimal Kumar, B. Cecil, B. M. Mangal Deep, P. Sivraj (2014) “Smart Soldier Assistance using WSN” International Conference on Embedded Systems - (ICES 2014), IEEE, pp: (244-249).

# [4] P.S. Kurhe, S.S. Agrawal (2013) “Real Time Tracking and Health Monitoring System of Remote Soldier Using ARM 7”, International Journal of Engineering Trends and Technology, 4(3), pp: (311-315).

# [5] Shruti Nikam, Supriya Patil, Prajkta Powar, V. S. Bendre (2013) “GPS Based Soldier Tracking and Health Indication System” International Journal of Advanced Research in Electrical, Electronics and Instrumentation Engineering, 2(3), pp: (1082-1088).

# [6] Prof. Pravin Wararkar, Sawan Mahajan, Ashu Mahajan, Arijit Banerjee, Anchal Madankar, Ashish Sontakke (2013) “Soldier Tracking and Health Monitoring System” The International Journal of Computer Science & Applications, 2(02), pp: (81-86).

# [7] Govindaraj A., Dr. S. Sindhuja Banu (2013) “GPS Based Soldier Tracking and Health Indication System with Environmental Analysis”, International Journal of Enhanced Research in Science Technology & Engineering, 2(12), pp: (46-52).

# [8] Palve Pramod, “GPS Based Advanced Soldier Tracking With Emergency Messages & Communication System” (2014) International Journal of Advance Research in Computer Science and Management Studies, 2(6), pp: (25-32).

# [9] Mr. Rajdeep Limbu, Prof. V. V. Kale (2014) “GPS Based Soldier Tracking and Health Monitoring System” International Journal for Technological Research in Engineering, 1(12), pp: (1485-1488).

# [10] Rubina.A.Shaikh (2012) “Real Time Health Monitoring System of Remote Patient Using Arm7” International Journal of Instrumentation, Control and Automation, 1(3-4), pp: (102-105).

# [11] Ekta Madhyan, Mahesh Kadam (2014) “A Unique Health Care Monitoring System Using Sensors and ZigBee Technology” International Journal of Advanced Research in Computer Science and Software Engineering, 4(6), pp: (501-509).

# [12] Dr. S. S. Riaz Ahamed (2009) “The Role of ZigBee Technology in Future Data Communication System” Journal of Theoretical and Applied Information Technology, 5(2), pp: (129-135).

# [13] Nisha Ashok Somani, Yash Patel (2012) “ZigBee: A Low Power Wireless Technology for Industrial Applications” International Journal of Control Theory and Computer Modeling, 2(3), pp: (27-33).

# [14] P. Rohitha, P. Ranjeet Kumar, Prof. N. Adinarayana, Prof. T. Venkat Narayana Rao (2012) “Wireless Networking through ZigBee Technology” International Journal of Advanced Research in Computer Science and Software Engineering, 2(7) , pp: (49-54).

# [15] Dineshkumar Jaiswar, Sanjna S. Repal (2015) “Real Time Tracking and Health Monitoring of Soldiers using ZigBee Technology: a Survey” International Journal of Innovative Research in Science, Engineering and Technology, 4(7).

# [16] Shruti Nikam, Supriya Patil, Prajkta Powar, V.S.Bendre (2013) “GPS based Soldier Tracking and Health Indication System” International Journal of Advanced Research in Electrical, Electronics and Instrumentation Engineering, 2(3).

# [17] Akshita V. Armarkar, Deepika J. Punekar, Mrunali V. Kapse, Sweta Kumari, Jayshree A. Shelke (2017) “Soldier Health and Position Tracking System” IJESC, 7(3).

# [18] Afef Mdhaffar, Tarak Chaari, Kaouthar Larbi, Mohamed Jmaiel and Bernd Freisleben (2017) “IoT-based Health Monitoring via LoRaWAN” IEEE EUROCON.

# [19] M.V.N.R. Pavan Kumar, Ghadge Rasika Vijay, Patil Vidya Adhikrao, Bobade Sonali Vijaykumar (2014) “Health Monitoring and Tracking of Soldier Using GPS” International Journal of Research in Advent Technology, 2(4).

# [20] R. Archana, S. Indira (2013) “Soldier Monitoring and Health Indication System” International Journal of Science and Research (IJSR)